SMART PARKING SYSTEM

USING ESP32 & ULTRASONIC SENSOR WITH LIVE WEB DASHBOARD

## A PROJECT REPORT

Submitted by

PATEL MANAN– 230203204038

PATEL SOYAVATI– 230203204039

In partial fulfilment for the award of

# Diploma

in

Computer Engineering

Vidhyadeep Institute of

Engineering and Technology

(Diploma)

Vidhyadeep University, Surat, Gujarat

## A.Y 2025-26

SMART PARKING SYSTEM

USING ESP32 & ULTRASONIC SENSOR WITH LIVE WEB DASHBOARD

Submitted by

PATEL MANAN– 230203204038

PATEL SOYAVATI– 230203204039

Department: Computer Engineering

Institute: Vidhyadeep Institute of Engineering and Technology (Diploma)

University: Vidhyadeep University, Anita-Kim, Surat, Gujarat

Academic Year: 2025–26

# VIDHYADEEP UNIVERSITY

VIDHYADEEP INSTITUTE OF ENGINEERING AND

TECHNOLOGY(DIPLOMA)

DECLARATION

We hereby declare that the Project report submitted along with the Project entitled SMART PARKING SYSTEM USING ESP32 & ULTRASONIC SENSOR WITH LIVE WEB DASHBOARD submitted in partial fulfilment for the degree of Diploma of Engineering in Computer Engineering to Vidhyadeep University, Anita, Kim, Surat, is a bonafide record of original project work carried out by us and internal guidance of Ms. Aeni Parmar. And that no part of this report has been directly copied from any students’ reports or taken from any other source, without providing

| Name Of Student | Enrollment Number | Signature Of Student |
| --- | --- | --- |
| Patel Manan | 230203204038 |  |
| Patel Soyavati | 230203204039 |  |

# VIDHYADEEP  UNIVERSITY

VIDHYADEEP INSTITUTE OF ENGINEERING AND

TECHNOLOGY(DIPLOMA)

CERTIFICATE

This is to certify that the project report submitted along with the project entitled “Smart Parking System Using ESP32 & Ultrasonic Sensor with Live Web Dashbord” has been carried out by Manan Patel (230203204038) , Soyavati Patel (230203204039) under my guidance in partial fulfilment for Diploma in Computer Engineering, 6th Semester of Vidhyadeep University, Anita-Kim, Surat during the academic year   2025-26.

Internal Guide                                                                                             Head of the Department

# ACKNOWLEDGEMENT

We would like to express Our deepest gratitude to everyone who contributed to the successful completion of this project. First and foremost, We extend our heartfelt thanks to our internal guide, Ms. Aeni Parmar, for their invaluable guidance, constant encouragement, and insightful feedback throughout the development of this project.

We sincerely thank Mr. Jaydeep Baria, Head of the Department, for the opportunity and continuous support, and Dr. Vikas J. Patel, our principal, for providing the resources and academic environment that made this project possible.

Lastly, we would like to acknowledge our family and friends for their unwavering support, patience, and encouragement throughout this journey

Yours Sincerely

Patel Manan (230203204038)

Patel Soyavati (230203204039)

# ABSTRACT

Indoor tracking has become an important requirement in modern environments where monitoring the location of people and assets is necessary. Most existing tracking systems rely on Global Positioning System (GPS) technology, which works effectively in outdoor environments. However, GPS signals are not reliable inside buildings, underground areas, or covered spaces, making indoor tracking a major challenge.

To overcome this limitation, this project presents an Indoor Tracking System that uses Bluetooth and LoRa communication technologies to accurately track devices inside buildings. In this system, mobile devices act as signal transmitters using applications such as nRF Connect, continuously broadcasting Bluetooth signals with unique device identifiers.

These signals are received by nodes installed in different locations such as rooms, floors, and buildings. Each node is built using an ESP32 microcontroller connected with an SX1278 (Ra-02) LoRa module. The nodes collect important data such as device ID, node ID, RSSI (signal strength), and timestamp, and transmit it to a central server using LoRa communication.

The server acts as the brain of the system. It processes the collected data using intelligent algorithms to determine the most accurate location of each device. The system uses techniques such as RSSI comparison, multi-node confirmation, and stability checking to improve accuracy and avoid incorrect location detection. The processed data is then stored in a MySQL database.

A user-friendly web-based dashboard is developed using HTML, CSS, JavaScript, and PHP to display the tracking data. The dashboard provides features such as real-time tracking (live mode), search functionality, and filtering based on building, floor, and class. This allows users to easily monitor and track specific devices or individuals.

To handle continuous data generation, the system includes a backup mechanism that automatically stores daily tracking data in structured JSON files. This helps in maintaining database performance and ensures safe storage of historical data.

This system can be applied in various environments such as schools, hospitals, offices, and shopping malls for tracking students, staff, and assets. In the future, the system can be enhanced using Ultra-Wideband (UWB) technology for higher accuracy and by integrating a map-based visualization system for better user experience.

In conclusion, the proposed Indoor Tracking System provides a cost-effective, scalable, and efficient solution for indoor location tracking where traditional GPS-based systems fail.

LIST OF FIGURES

FIGURE 1.1 Scope Flow Representation 	04

FIGURE 2.1 System Architecture Representation	10

FIGURE 2.2 System Block Diagram	10

FIGURE 2.3 System Flow Chart	11

FIGURE 3.1 Module Interaction Overview	20

FIGURE 3.2 Flow Representation of Algorithm	21

FIGURE 3.2 5.1 Web-based dashboard visualization	35

LIST OF TABLES

TABLE 1.1 Scope 	03

TABLE 1.1 Primary Objectives	04

TABLE 2.1Hardware Requiment	07

TABLE 2.2 Software Requiment	08

TABLE 2.3 Praking system comperition	13

TABLE 3.1 Program Language	15

TABLE 3.2 Software Tools	16

TABLE 3.3 Test Environment	17

TABLE 3.4 Faults and Remedies	22

TABLE 4.1 ESP32 Initialization Test Case	24

TABLE 4.2 Ultrasonic Sensor Distance Measurement Test Case	24

TABLE 4.3 Parking Slot Occupancy Detection Test Case	25

TABLE 4.4 Led Status Indication Test Case	25

TABLE 4.5 Wi-Fi Connectivity Test Case	26

TABLE 4.6 Server Data Update Test Case	26

TABLE 4.7 Live Dashboard Display Test Case	27

TABLE 4.8 Performance Evaluation Table	30

LIST OF ABBREVIATION

| ABBREVIATION | FULL FORM |
| --- | --- |
| ESP32 | Espressif 32-bit Microcontroller |
| IoT | Internet of Things |
| Wi-Fi | Wireless Fidelity |
| GPIO | General Purpose Input/Output |
| LED | Light Emitting Diode |
| HC-SR04 | Ultrasonic Distance Measuring Sensor |
| MCU | Microcontroller Unit |
| IDE | Integrated Development Environment |
| IP | Internet Protocol |
| UI | User Interface |
| DB | Database |
| RAM | Random Access Memory |
| CPU | Central Processing Unit |
| HTML | HyperText Markup Language |
| CSS | Cascading Style Sheets |
| PHP | Hypertext Preprocessor |
| SQL | Structured Query Language |
| AC | Alternating Current |
| DC | Direct Current |

TABLE OF CONTENTS

ACKNOWLEDGEMENT	i

ABSTRACT	ii

LIST OF FIGURES	iii

LIST OF TABLES	iv

LIST OF ABBREVIATION	v

CHAPTER 1: INTRODUCTION	01

  - PROBLEM STATEMENT AND PROJECT SUMMARY	01

  - PURPOSE AND SCOPE OF THE PROJECT	02

  - OBJECTIVES OF THE PROJECT	04

CHAPTER 2: SYSTEM ANALYSIS AND DESIGN	06

  - LITERATURE REVIEW	06

  - TECHNOLOGY	07

  - REQUIREMENT ANALYSIS	07

  - SYSTEM DESIGN	08

  - TECHNOLOGY AND LITERATURE REVIEW	12

CHAPTER 3: SYSTEM IMPLEMENTATION	15

  - IMPLEMENTATION ENVIRONMENT	15

  - SYSTEM IMPLEMENTATION	18

  - FAULTS AND REMEDIES	22

CHAPTER 4: RESULTS AND DISCUSSION	23

  - TESTING AND RESULTS	23

CHAPTER 5: CONCLUSION AND FUTURE WORK	31

  - CONCLUSION	31

  - FUTURE SCOPE	32

CHAPTER 6: BIBLIOGRAPHY AND REFERENCES	36

CHAPTER 7: FUTURE ENHANCEMENTS / RECOMMENDATIONS	39

  - INTRODUCTION	39

  - FUTURE ENHANCEMENTS	39

  - CONCLUSION	41

APPENDIX	42

# CHAPTER 1: INTRODUCTION

In today’s rapidly urbanizing world, the number of vehicles is increasing at an unprecedented rate. As cities grow denser, parking management has emerged as a serious challenge for residential societies, commercial complexes, educational institutions, hospitals, and shopping malls. Traditional parking systems rely heavily on manual observation and human intervention, which often leads to inefficiency, congestion, fuel wastage, and user frustration.

With the advancement of Internet of Things (IoT) and embedded systems, it is now possible to automate parking monitoring and provide real-time information to users. Smart parking solutions aim to reduce the time spent searching for parking spaces, improve space utilization, and enhance the overall user experience. This project focuses on designing an intelligent, cost-effective, and scalable smart parking system that can be easily deployed in real-world environments.

## PROBLEM STATEMENT AND PROJECT SUMMARY

#### PROBLEM STATEMENT

Parking-related issues are a daily concern in urban and semi-urban areas. Drivers often spend a significant amount of time searching for an available parking slot, especially during peak hours. This not only causes inconvenience but also contributes to traffic congestion, increased fuel consumption, and environmental pollution.

Most existing parking systems suffer from the following limitations:

- Lack of real-time parking availability information

- Dependence on manual monitoring or security staff

- Inefficient use of available parking space

- High installation and maintenance cost of advanced systems

- No centralized monitoring or data logging

#### Core Problem Definition

To design and develop a smart parking system that can automatically detect parking slot availability and display real-time status to users using a simple, reliable, and low-cost technological_solution

#### 1.1.2 PROJECT SUMMARY

he proposed project presents a Smart Parking System that uses sensor-based detection and wireless communication to monitor parking slot occupancy in real time. The system continuously checks whether a parking slot is occupied or vacant and updates this information on a live digital interface.

## How the System Works:-

## 1. A sensor is installed in each parking slot to detect the presence of a vehicle.

## 2. The sensor measures distance and determines whether a car is parked.

## 3. A microcontroller processes this data and decides the slot status.

## 4. The status is sent wirelessly to a server or dashboard.

## 5. Users can instantly see which slots are free or occupied.

## This approach eliminates the need for manual checking and provides accurate, real-time information to users.

## PURPOSE AND SCOPE OF THE PROJECT

The success of any engineering project depends on a clear understanding of why the system is developed and what its operational boundaries are. This section explains the purpose behind developing the proposed smart parking system and defines the scope within which the project operates.

#### PURPOSE

he primary purpose of this project is to design and implement an intelligent parking management system that simplifies the process of identifying available parking spaces. In conventional parking areas, users rely on visual inspection or manual assistance, which is inefficient and time-consuming.

This project aims to:

  - Provide real-time parking availability information without human intervention

  - Reduce the effort and time required to search for parking spaces

  - Improve parking space utilization through automation

  - Demonstrate the practical application of IoT and embedded systems in solving real- world problems

  - Develop a system that is simple, affordable, and easy to deploy

From an academic perspective, the project also serves as a learning platform for understanding sensor interfacing, wireless communication, server interaction, and realtime data visualization.

In essence, the purpose of this project is to bridge the gap between traditional parking systems and modern smart city requirements by offering a reliable and user-friendly automated solution.

#### SCOPE

The scope of the project defines the functional coverage and limitations of the proposed system. The smart parking system is designed with scalability in mind, allowing it to be expanded based on future requirements.

| Area | Description |
| --- | --- |
| Slot Detection | Automatic detection of vehicle presence in parking slots |
| Real-Time Updates | Live status display of parking availability |
| Wireless Communication | Data transmission using Wi-Fi |
| Monitoring Interface | Simple and visual dashboard for users |
| Expandability | Support for multiple parking slots |
| Cost Efficiency | Uses low-cost components |

TABLE 1.1 Scope

System Coverage

- Suitable for small to medium-sized parking areas

- Can be deployed in indoor and semi-outdoor environments

- Supports continuous monitoring without manual checking

- Designed to work with minimal maintenance

Out-of-Scope (Current Version)

- Automated vehicle guidance or navigation

- Payment or billing systems

- License plate recognition

- Cloud-based analytics (can be added in future)

Parking flow:

## FIGURE 1.1 Scope Flow Representation

## 1.3 OBJECTIVES OF THE PROJECT

The objectives define the specific and measurable goals that the project aims to achieve. These objectives guided the system design, development, and testing phases.

### PRIMARY OBJECTIVES

| Objective No. | Objective Description |
| --- | --- |
| 1 | To design an automated system for detecting parking slot occupancy |
| 2 | To provide real-time parking availability information |
| 3 | To minimize manual monitoring and human dependency |
| 4 | To reduce time, fuel consumption, and traffic congestion |
| 5 | To develop a low-cost and scalable solution |

TABLE 1.2 Primary Objectives

TECHNICAL OBJECTIVES

- Interface sensors with a microcontroller for distance measurement

- Process sensor data to classify slot status accurately

- Transmit parking data using wireless communication

- Display data on a real-time monitoring interface

### USER-CENTRIC OBJECTIVES

- Make the system easy to understand for non-technical users

- Present information in a visually clear and simple manner

- Improve overall parking experience and user satisfaction

### EDUCATIONAL OBJECTIVES

- Apply theoretical knowledge to a real-world problem

- Gain practical exposure to IoT system design

- Understand system integration of hardware and software components

# CHAPTER 2: SYSTEM ANALYSIS AND DESIGN

System analysis and design is a vital stage in project development, as it defines how the system will function and how different components interact with each other. This chapter discusses the study of existing work, technologies used, system requirements, and the overall design of the proposed smart parking system.

## LITERATURE REVIEW

A literature review provides an understanding of the existing research, technologies, and methods related to the problem domain. In the field of parking management systems, numerous studies have been conducted to address issues such as traffic congestion, inefficient space utilization, and time wastage caused by manual parking methods. Researchers have explored different approaches to automate parking slot detection and provide real-time parking information.

Early parking systems mainly relied on manual supervision, where security personnel guided vehicles to available slots. Although simple, these systems were inefficient, errorprone, and unsuitable for large parking areas. To overcome these limitations, automated parking solutions were introduced using various sensing and communication technologies.

Several research works have utilized Infrared (IR) sensors to detect vehicle presence. While IR-based systems are inexpensive, their performance is affected by environmental factors such as sunlight, dust, and temperature variations. To improve accuracy, many studies proposed the use of ultrasonic sensors, which measure distance based on sound waves and provide more reliable results under different lighting conditions.

Camera-based parking systems using image processing and computer vision have also been studied extensively. These systems offer high accuracy and advanced features but require complex algorithms, high processing power, and expensive hardware, making them costly to implement and maintain. Similarly, RFID-based parking systems enable vehicle identification but require RFID tags and readers, increasing infrastructure cost.

Recent research emphasizes the role of Internet of Things (IoT) in smart parking systems. IoT-based solutions integrate sensors, embedded controllers, wireless communication, and web or mobile interfaces to provide real-time monitoring. Studies indicate that IoT-enabled parking systems significantly reduce parking search time and improve user convenience. However, many cloud-based IoT systems depend heavily on continuous internet connectivity and incur additional operational costs.

From the reviewed literature, it is evident that there is a need for a simple, low-cost, and reliable smart parking system that provides real-time information without complex infrastructure. The proposed project is inspired by these studies and aims to combine the advantages of sensor-based detection and IoT technology while minimizing cost and system complexity.

## TECHNOLOGY

The proposed smart parking system uses modern embedded and IoT technologies to automate parking slot detection and monitoring. The key technologies used in the system include:

- Sensor technology for vehicle detection

- Embedded systems for data processing

- Wireless communication for real-time data transfer

- Web-based technologies for visualization

These technologies were selected to ensure low cost, reliability, scalability, and ease of implementation.

## REQUIREMENT ANALYSIS

Requirement analysis identifies all necessary components needed to develop the system successfully. It helps in understanding both hardware and software needs before system implementation.

### HARDWARE REQUIREMENTS

Hardware components are responsible for detecting vehicle presence, processing data, and indicating parking status.

| Component | Purpose |
| --- | --- |
| Ultrasonic Sensor | Detects vehicle presence by measuring distance |
| Microcontroller | Processes sensor data and controls system logic |
| Wi-Fi Module | Enables wireless communication |
| LED Indicators | Shows parking slot status |
| Jumper Wires | Used for electrical connections |
| Power Supply | Provides required voltage |

TABLE 2.1Hardware Requiment

Reason for Hardware Selection:

- Low power consumption

- High detection accuracy

- Easy availability

- Cost-effective

### 2.3.2 SOFTWARE REQUIREMENTS

| Software | Purpose |
| --- | --- |
| Embedded IDE | Used for programming the controller |
| Server-Side Language | Handles data reception |
| Database System | Stores parking status information |
| Web Technologies | Used for dashboard design |
| Communication Protocol | Enables data transfer |

Software components control data processing, communication, storage, and user interface.

TABLE 2.2 Software Requiment

These software tools ensure real-time data handling and smooth system operation

## 2.4 SYSTEM DESIGN

System design explains how the system components are organized and how data flows from input to output. A well-structured design improves system reliability and performance.

### SYSTEM ARCHITECTURE

System architecture defines the overall structure of the proposed system and explains how different components interact to achieve the desired functionality. The architecture of the smart parking system is designed to be simple, modular, and scalable, ensuring reliable real-time monitoring with minimum complexity.

The proposed system follows a layered architecture, where each layer performs a specific function and communicates with the next layer in a well-defined manner.

#### Architectural Layers Description

The system architecture is divided into three main layers:

#### Sensing Layer

This layer is responsible for detecting the presence of vehicles in parking slots.

  - Sensors are installed in each parking slot

  - The sensor continuously measures distance

  - Based on the measured value, vehicle presence is identified This layer acts as the input unit of the system.

#### Processing and Communication Layer

This layer acts as the core of the system.

  - Sensor data is received by the microcontroller

  - The microcontroller processes the data and determines slot status

  - The slot is classified as Occupied or Vacant

  - Processed data is transmitted wirelessly using Wi-Fi

This layer ensures accurate decision-making and real-time data transfer.

#### Application Layer

This layer provides the user interface and data visualization.

  - Parking status data is received by the server

  - Information is stored in a database

  - A dashboard displays real-time parking availability

  - Users can easily view slot status

This layer focuses on user interaction and monitoring

[Sensing Layer]

Vehicle Detection

[Processing Layer]

Data Analysis

[Communication Layer]

Wireless

Transfer

[Application Layer]

Live Status Display

FIGURE 2.1 System Architecture Representation

2.4.2 BLOCK DIAGRAM

The block diagram shows the functional interaction between system components:

FIGURE 2.2 System Block Diagram

This design ensures efficient and real-time monitoring of parking slots.

### 2.4.3 FLOW CHART

The operational flow of the system is represented below:

FIGURE 2.3 System Flow Chart

## TECHNOLOGY AND LITERATURE REVIEW

This section presents a combined discussion of the technologies selected for the proposed system and the supporting literature that justifies their use. By linking practical technology choices with findings from previous research, this section validates the design decisions made in the project.

### TECHNOLOGY USED

The proposed smart parking system is developed using modern embedded system and IoT technologies that are widely accepted in real-world applications. The selection of each technology was based on criteria such as accuracy, reliability, cost-effectiveness, and ease of implementation.

The core technologies used in the system include:

- Sensor-based detection for identifying vehicle presence

- Embedded microcontroller technology for real-time data processing

- Wireless communication for instant data transmission

- Web-based interface for live monitoring and visualization

Sensor-based systems are preferred because they provide fast response time and do not require complex image processing. Embedded controllers enable continuous monitoring and decision-making with low power consumption. Wireless communication allows the system to update parking status in real time without extensive wiring. The web-based dashboard ensures that parking information is displayed in a simple and user-friendly manner.

Together, these technologies create a system that is efficient, scalable, and suitable for practical deployment.

#### Comparison with Existing Systems

To evaluate the effectiveness of the proposed smart parking system, it is important to compare it with existing parking management approaches. This comparison highlights how the proposed system improves upon traditional and previously developed automated systems in terms of cost, accuracy, complexity, and usability.

Existing parking systems can generally be classified into manual systems, basic automated systems, and advanced smart systems. Each category has its own strengths and limitations

### COMPARISON OF PARKING SYSTEMS

| Parameter | Manual Parking System | Existing Automated Systems | Proposed Smart Parking System |
| --- | --- | --- | --- |
| Parking Detection | Manual observation | IR sensors / Cameras / RFID | Ultrasonic sensor-based detection |
| Real-Time Information | Not available | Limited or delayed | Fully real-time updates |
| Accuracy | Low (human error) | Moderate to high | High and reliable |
| Installation Cost | Low | High | Low |
| System Complexity | Very simple | Complex | Simple and modular |
| Maintenance | High (human dependency) | High | Low |
| Scalability | Difficult | Limited | Easy to expand |
| User Interface | Not available | Complex or appbased | Simple and userfriendly |
| Internet Dependency | Not required | High | Low to moderate |

TABLE 2.3 Praking system comperition

#### Analysis of Comparison

Manual parking systems are easy to implement but are highly inefficient and unreliable, especially during peak hours. Existing automated systems improve accuracy but often involve expensive hardware such as cameras or RFID infrastructure, making them unsuitable for small and medium parking facilities.

The proposed smart parking system offers a balanced solution by combining low-cost sensor-based detection with real-time monitoring and a simple system architecture. Unlike camera-based systems, it does not require complex image processing, which reduces system cost and computational load. Compared to IR-based systems, ultrasonic sensors provide better accuracy under varying environmental condition

2.5.3 LITERATURE SUPPORT

Various research studies and technical surveys support the use of sensor-based smart parking systems. Literature indicates that traditional manual parking systems are inefficient and fail to provide real-time information. Researchers have shown that automated systems significantly reduce parking search time and traffic congestion.

Previous studies highlight that:

      - Ultrasonic sensors offer better reliability compared to IR sensors in different lighting conditions

      - Embedded system-based solutions provide stable and continuous operation

      - Local or server-based monitoring reduces response delay

      - Simple system architectures improve system maintainability

Several research papers also emphasize the importance of real-time data visualization and low-cost implementation, especially for small and medium parking areas. The proposed project aligns with these findings by adopting proven technologies while focusing on simplicity and affordability.

# CHAPTER 3: SYSTEM IMPLEMENTATION

System implementation is the phase where the theoretical design of the system is transformed into a working model. This chapter explains the environment in which the system was implemented, the tools and technologies used, the hardware setup, and the step- by-step implementation of the smart parking system. It also discusses possible faults encountered during development and their remedies.

## IMPLEMENTATION ENVIRONMENT

The implementation environment refers to the complete setup in which the proposed smart parking system was developed, integrated, tested, and validated. It includes the selection of programming languages, software tools, hardware configuration, and testing conditions. A properly defined implementation environment ensures that the system performs reliably, produces consistent results, and can be easily replicated or enhanced in the future.

The environment was carefully designed to support real-time data processing, wireless communication, and user-friendly monitoring, which are essential requirements for a smart parking system.

### PROGRAMMING LANGUAGES

Multiple programming languages were used to develop different parts of the system. Each language was selected based on its suitability for a specific task.

| PROGRAMMING LANGUAGE | ROLE IN THE SYSTEM |
| --- | --- |
| Embedded C / C++ | Used to program the microcontroller, handle sensor data, and control logic |
| HTML | Used to structure the web-based parking dashboard |
| CSS | Used to improve the visual appearance and readability of the dashboard |
| PHP | Used for server-side processing and handling data received from the controller |
| SQL | Used for storing and retrieving parking slot data from the database |

TABLE 3.1 Program Language

#### Detailed Explanation:

      - Embedded C/C++ allows efficient control of hardware components such as sensors and LEDs. It ensures fast execution and low memory usage, which is critical for embedded systems.

      - HTML and CSS make the dashboard simple, clear, and visually attractive so that users can easily understand parking status.

      - PHP acts as a bridge between the hardware system and the database by processing incoming data.

      - SQL enables structured storage of parking information such as slot status and timestamps.

### SOFTWARE TOOLS

Several software tools were used during the development, testing, and debugging of the system.

| SOFTWARE TOOL | PURPOSE |
| --- | --- |
| Embedded Development IDE | Writing, compiling, and uploading code to the microcontroller |
| Local Server Software | Hosting PHP scripts and database locally |
| Database Management System | Storing parking slot status and updates |
| Web Browser | Viewing and testing the dashboard interface |
| Serial Monitor | Displaying real-time debugging messages |

#### TABLE 3.2 Software Tools

#### Detailed Explanation:

      - The Embedded IDE simplifies firmware development and provides debugging support.

      - The local server allows fast testing without dependency on external internet services.

      - The database system ensures structured and reliable data storage.

      - The web browser helps verify dashboard functionality from a user perspective.

      - The serial monitor is useful for detecting errors and validating sensor readings.

### HARDWARE SETUP

The hardware setup forms the physical foundation of the smart parking system. It includes sensors, a microcontroller, indicator devices, and a power source.

#### Hardware Configuration Details:

      - Each parking slot is equipped with a distance-measuring sensor

      - Sensors are mounted at an appropriate height to detect vehicles accurately

      - Sensor outputs are connected to the microcontroller

      - LED indicators are connected to display slot status locally

      - A regulated power supply is used to ensure stable operation

      - Wireless communication is enabled for data transmission

This configuration ensures accurate detection, real-time response, and safe operation

of the system.

### ENVIRONMENTAL CONDITIONS FOR TESTING

To evaluate the reliability and performance of the system, testing was conducted under various environmental conditions.

| CONDITION | TESTING DESCRIPTION |
| --- | --- |
| Indoor Testing | Controlled lighting and stable temperature |
| Semi-Outdoor Testing | Exposure to natural light and airflow |
| Distance Variation | Different vehicle sizes and parking positions |
| Network Conditions | Stable and moderate Wi-Fi signal |

#### TABLE 3.3 Test Environment

#### Testing Significance:

      - Indoor testing ensures controlled performance evaluation

      - Semi-outdoor testing checks sensor reliability under real conditions

      - Distance variation testing validates detection accuracy

      - Network testing ensures smooth real-time data transmission

These tests confirm that the system performs reliably under different conditions.

## SYSTEM IMPLEMENTATION

System implementation refers to the actual execution of the proposed design by integrating hardware and software components into a working system. In this phase, the smart parking system is developed by configuring sensors, programming the controller, establishing wireless communication, and implementing the monitoring interface.

The implementation was carried out in a modular manner, which improves system clarity, simplifies debugging, and allows future enhancements without redesigning the entire system.

### MODULE-WISE EXPLANATION

The smart parking system is divided into several functional modules. Each module performs a specific task and works together to achieve real-time parking monitoring.

#### Sensor Module

The sensor module is responsible for detecting the presence of a vehicle in the parking slot.

- A distance-measuring sensor is placed in each parking slot

- The sensor continuously measures the distance between itself and any object

- When a vehicle enters the slot, the measured distance decreases

- The sensor sends distance data to the processing module

### PURPOSE

To provide accurate and continuous input regarding vehicle presence.

#### Processing Module

The processing module acts as the brain of the system.

- Receives distance values from the sensor

- Compares measured distance with a predefined threshold

- Determines whether the slot is Occupied or Vacant

- Controls LED indicators based on slot status Decision Logic Example:

- Distance ≤ threshold → Slot Occupied

- Distance > threshold → Slot Vacant

#### Communication Module

This module enables real-time data transmission between the parking system and the monitoring interface.

- Transmits parking status wirelessly

- Sends updated slot information only when a change is detected

- Reduces unnecessary data transmission and network load

### PURPOSE

To ensure fast and reliable real-time updates.

#### Server and Database Module

This module handles data storage and management.

- Receives parking slot data

- Stores status information with timestamps

- Maintains a record of parking activity

#### Benefits:

- Enables real-time monitoring

- Supports future data analysis

- Improves system reliability

#### Display Module

This module provides the user interface.

- Displays parking slot status clearly

- Uses simple visual indicators

- Updates automatically without manual refresh

### PURPOSE

To make parking information easy to understand for all users.

Module Interaction Overview

FIGURE 3.1 Module Interaction Overview

### ALGORITHMS / PROCESS STEPS

The system follows a simple and efficient algorithm to ensure continuous and accurate operation.

#### Step-by-Step Algorithm

- Start the system

- Initialize all hardware components

- Read distance values from the sensor

- Compare distance with predefined threshold

- If distance indicates vehicle presence:

  - Mark slot as Occupied

- Else:

  - Mark slot as Vacant

- Update LED indicators accordingly

- Transmit slot status to server

- Store status in database

- Display updated status on dashboard

- Repeat the process continuously

#### Flow Representation of Algorithm

## FIGURE 3.2 Flow Representation of Algorithm

## FAULTS AND REMEDIES

During system implementation and testing, certain faults were observed that affected the performance of the smart parking system. Identifying these faults and applying appropriate remedies helped improve system reliability and stability. The table below lists the common issues encountered along with their possible causes and remedies.

| Fault | Possible Cause | Remedy |
| --- | --- | --- |
| Incorrect detection | Improper sensor placement or alignment | Adjust sensor position and calibration |
| No data transmission | Network connectivity issue | Check and reconnect Wi-Fi connection |
| Delayed updates | High server load or frequent data requests | Optimize server-side code |
| LED not working | Loose or faulty wiring | Reconnect or replace wiring |
| System reset | Power fluctuation or unstable supply | Use a stable and regulated power supply |

TABLE 3.4 Faults and Remedies

This concise representation clearly highlights practical challenges faced durinimplementation and demonstrates effective problem-solving, which is important for realworld deployment and academic evaluation.

# CHAPTER 4: RESULTS AND DISCUSSION

This chapter presents a detailed evaluation of the proposed smart parking system through systematic testing and analysis. The objective of testing was to verify whether the system meets the functional requirements defined during the design phase and performs reliably under real-world conditions. The results obtained from testing are analyzed and discussed to assess system accuracy, responsiveness, stability, and overall effectiveness.

## TESTING AND RESULTS

Testing and result analysis is a crucial phase in validating the correctness, efficiency, and reliability of the proposed smart parking system. The primary objective of testing was to ensure that the implemented system performs according to the design specifications and meets all functional and performance requirements defined during system analysis.

After complete integration of hardware and software components, the system was tested under controlled as well as real-world conditions. Various parking scenarios were created to evaluate how accurately the system detects vehicle presence, how efficiently it communicates data, and how reliably it displays real-time parking information.

The testing process focused on the following critical parameters:

- Accuracy of vehicle detection

- Correctness of slot status classification

- Real-time data transmission performance

- Stability during continuous operation

- User interface clarity and responsiveness

### TEST CASES

      - ESP32 INITIALIZATION TEST CASE

Test Case Name: ESP32 Initialization Test Case Test Case Description:

To verify that the ESP32 initializes correctly and establishes communication with the ultrasonic sensor, Wi-Fi module, and server.

#### Test Case Status : Pass Test Scenario :

Verify that ESP32 boots successfully, connects to Wi-Fi, and prepares the system for parking slot detection without errors.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Power ON ESP32 | ESP32 boots successfully | As Expected | Pass |
| 2 | Initialize peripherals | Ultrasonic sensor & LEDs initialized | As Expected | Pass |
| 3 | Check serial output | No error messages displayed | As Expected | Pass |

TABLE 4.1 ESP32 Initialization Test Case

### ULTRASONIC SENSOR DISTANCE MEASUREMENT TEST CASE

Test Case Name: Ultrasonic Sensor Distance Measurement Test Case Test Case Description:

To verify accurate distance measurement using the ultrasonic sensor.

#### Test Case Status: Pass Test Scenario:

Verify that the ultrasonic sensor correctly measures distance between the sensor and vehicle.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | No vehicle in slot | Distance > threshold | As Expected | Pass |
| 2 | Place vehicle in slot | Distance < threshold | As Expected | Pass |
| 3 | Remove vehicle | Distance returns to normal | As Expected | Pass |

### TABLE 4.2 Ultrasonic Sensor Distance Measurement Test Case

### PARKING SLOT OCCUPANCY DETECTION TEST CASE

Test Case Name: Parking Slot Occupancy Detection Test Case Test Case Description:

To verify correct detection of parking slot status (Occupied / Vacant).

#### Test Case Status: Pass Test Scenario:

Verify that the system correctly identifies whether a parking slot is occupied or vacant.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Slot empty | Status = Vacant | As Expected | Pass |
| 2 | Vehicle parked | Status = Occupied | As Expected | Pass |
| 3 | Vehicle removed | Status = Vacant | As Expected | Pass |

TABLE 4.3 Parking Slot Occupancy Detection Test Case

### LED STATUS INDICATION TEST CASE

Test Case Name: LED Status Indication Test Case Test Case Description:

To verify correct LED indication based on parking slot status.

#### Test Case Status: Pass Test Scenario:

Verify that LEDs correctly indicate slot availability.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Slot vacant | Green LED ON | As Expected | Pass |
| 2 | Slot occupied | Red LED ON | As Expected | Pass |

### TABLE 4.4 Led Status Indication Test Case

### WI-FI CONNECTIVITY TEST CASE

Test Case Name: Wi-Fi Connectivity Test Case Test Case Description:

To verify that ESP32 connects to the Wi-Fi network successfully.

#### Test Case Status: Pass Test Scenario:

Verify that ESP32 establishes a stable Wi-Fi connection for data transmission.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Power ON ESP32 | Wi-Fi connection initiated | As Expected | Pass |
| 2 | Network available | ESP32 connected | As Expected | Pass |
| 3 | Check IP address | Valid IP assigned | As Expected | Pass |

TABLE 4.5 Wi-Fi Connectivity Test Case

### SERVER DATA UPDATE TEST CASE

Test Case Name: Server Data Update Test Case Test Case Description:

To verify that parking slot data is sent to the server and stored correctly.

#### Test Case Status: Pass Test Scenario:

Verify that slot status updates are transmitted and stored in the database.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Slot status changes | Data sent to server | As Expected | Pass |
| 2 | Database update | Status stored | As Expected | Pass |
| 3 | Refresh dashboard | Updated status shown | As Expected | Pass |

TABLE 4.6 Server Data Update Test Case

### LIVE DASHBOARD DISPLAY TEST CASE

Test Case Name: Live Dashboard Display Test Case Test Case Description:

To verify real-time display of parking slot status on the web dashboard.

#### Test Case Status: Pass Test Scenario:

Verify that dashboard reflects real-time parking availability.

| Step | Step Details | Expected Results | Actual Results | Pass / Fail / Not Executed |
| --- | --- | --- | --- | --- |
| 1 | Slot vacant | Dashboard shows Vacant | As Expected | Pass |
| 2 | Slot occupied | Dashboard shows Occupied | As Expected | Pass |
| 3 | Continuous update | Auto refresh works | As Expected | Pass |

TABLE 4.7 Live Dashboard Display Test Case

### TESTING METHODOLOGY

A structured and layered testing methodology was followed to ensure complete system validation.

#### Testing Levels Adopted

- Unit Testing

Each	module	(sensor,	processing,	communication,	display)	watested independently to verify its individual functionality.

#### Integration Testing

All modules were integrated and tested together to ensure correct data flow and interaction between hardware and software components.

#### System Testing

The complete system was tested in a real parking environment to validate end-to-end functionality.

#### Testing Procedure

  - Hardware wiring verification

  - Sensor calibration and threshold tuning

  - Validation of decision-making logic

  - Wireless data transmission testing

  - Dashboard response verification

  - Long-duration stability testing

#### Justification:

This methodology ensures early fault detection, minimizes system errors, and improves overall system robustness.

### MODULE-WISE TESTING AND OBSERVATIONS

Each module was tested individually to analyze its behavior and performance.

#### Sensor Module

- Provided stable and accurate distance readings

- Correctly detected vehicles of different sizes

- Minimal noise and fluctuation observed

#### Processing Module

- Correctly classified slot status based on threshold logic

- No incorrect decision during testing

- Fast processing with negligible delay

#### Communication Module

- Real-time data transmission achieved

- Data sent only on status change, reducing network load

- No packet loss observed

#### Server and Database Module

- Slot data stored accurately

- Timestamp updates verified

- Quick data retrieval observed

#### Display Module

- Dashboard updated automatically

- Clear visual representation of slot status

- Easy to understand for non-technical users

### OVERALL SYSTEM PERFORMANCE

The overall system performance of the proposed Smart Parking System was evaluated after completing all functional and module-wise testing. Performance evaluation focused on key parameters such as detection accuracy, response time, system stability, communication reliability, and user interface effectiveness.

#### Detection Accuracy

The ultrasonic sensor accurately detected the presence and absence of vehicles in the parking slot. During testing, the system consistently classified the slot status as Occupied when a vehicle was present and Vacant when the slot was empty. No false detections were observed under normal operating conditions, indicating high sensing accuracy.

#### Response Time

The system demonstrated near real-time response. As soon as the vehicle entered or exited the parking slot, the ESP32 processed the sensor data and updated the slot status almost instantly. The updated status was transmitted to the server and reflected on the dashboard with minimal delay, ensuring real-time monitoring.

#### System Stability

The system was tested for continuous operation over an extended period. The ESP32 remained stable without unexpected resets or crashes. All connected components functioned reliably, confirming that the system is suitable for long-term deployment in real parking environments.

#### Communication Reliability

Wi-Fi communication between the ESP32 and the server remained stable during testing. Slot status updates were transmitted successfully, and data loss was not observed. The system was designed to send updates only when a status change occurred, which optimized network usage and improved reliability.

#### User Interface Performance

The web-based dashboard displayed parking slot information clearly and accurately. The status indicators were easy to understand for both technical and non-technical users. Automatic refreshing ensured that the displayed data always reflected the current parking condition.

Performance Evaluation Table

| Performance Parameter | Observation |
| --- | --- |
| Detection Accuracy | High and consistent |
| Response Time | Near real-time |
| System Stability | Stable during long operation |
| Communication Reliability | Reliable data transmission |
| Dashboard Usability | Simple and user-friendly |

TABLE 4.8 Performance Evaluation Table

### RESULT SUMMARY

The results obtained from comprehensive testing confirm that the proposed Smart Parking System meets all functional and performance objectives defined during the design phase. The system successfully integrates hardware and software components to provide an automated, real-time parking management solution.

The ultrasonic sensor effectively detected vehicle presence, and the ESP32 processed sensor data accurately. Parking slot status updates were transmitted reliably through WiFi and stored on the server. The live dashboard displayed accurate and up-to-date information, improving user convenience and reducing the need for manual parking inspection.

Overall, the system proved to be:

- Accurate in detecting parking slot occupancy

- Fast in updating real-time parking status

- Stable during continuous operation

- Cost-effective and easy to deploy

- Scalable for multiple parking slots

The successful test results validate the practicality and reliability of the proposed smart parking solution. The system is suitable for deployment in locations such as shopping malls, residential complexes, office buildings, and educational institutions, contributing to efficient parking management and smart.

# CHAPTER 5: CONCLUSION AND FUTURE WORK

This chapter presents a detailed conclusion of the proposed Smart Parking System and outlines the potential future enhancements that can further improve its performance, scalability, and usability. The chapter highlights the practical significance of the project and its relevance in modern smart city environments.

## CONCLUSION

The Smart Parking System project successfully demonstrates the design and implementation of an IoT-based automated parking management solution. The system integrates an ESP32 microcontroller, ultrasonic sensor, Wi-Fi communication, and a web-based dashboard to monitor parking slot availability in real time.

The system is capable of:

- detecting the presence or absence of vehicles in parking slots

- automatically identifying slot status as Occupied or Vacant

- transmitting real-time parking data to a server using Wi-Fi

- displaying live parking availability on a user-friendly dashboard

- reducing manual effort and time required to search for parking

The implementation results indicate that the proposed system is accurate, reliable, costeffective, and easy to deploy. It significantly reduces traffic congestion, fuel wastage, and driver frustration, especially in high-demand parking areas such as malls, offices, hospitals, and educational institutions.

The modular design ensures easy scalability, allowing additional parking slots to be integrated without major system changes. The web-based interface makes the system accessible even to non-technical users.

Through this project, valuable knowledge and practical experience were gained in:

- embedded systems programming

- sensor interfacing and calibration

- IoT-based communication

- real-time data transmission

- server and database handling

- hardware–software integration

Overall, the Smart Parking System provides an efficient and practical solution to modern urban parking challenges. The project demonstrates how IoT technology can improve everyday infrastructure and contribute to smart city development.

## FUTURE SCOPE

Although the Smart Parking System successfully meets its current design objectives, several opportunities exist for further enhancement and expansion. Future developments can transform the prototype into a more intelligent, large-scale, and commercially viable parking solution.

#### Mobile Application Integration

A dedicated mobile application can be developed to enhance user convenience.

  - Real-time parking availability on smartphones

  - Push notifications instead of manual checking

  - Slot reservation before arrival

  - Navigation assistance to available slots

#### Cloud Storage and Data Analytics

Integration with cloud platforms can improve scalability and data utilization.

  - Secure cloud-based parking data storage

  - Analysis of peak parking hours

  - Parking usage pattern identification

  - Support for smart decision-making by authorities

#### Intelligent Automation and Analytics

Advanced algorithms can make the system smarter and more adaptive.

  - Prediction of parking availability

  - Dynamic allocation of parking slots

  - Reduction of false detections

  - Intelligent threshold adjustment

#### Improved Hardware Design

The physical deployment can be optimized for durability and outdoor use.

  - Weather-resistant sensor enclosures

  - Compact and robust mounting design

  - Industrial-grade ultrasonic sensors

  - Reduced wiring complexity

#### Power Optimization and Energy Efficiency

Energy efficiency can be improved for long-term operation.

  - Low-power modes in ESP32

  - Solar-powered parking nodes

  - Intelligent power management

  - Battery backup for power failure

#### Automated Billing and Access Control

The system can be extended to support complete parking automation.

  - Online payment integration

  - Time-based parking charges

  - Entry–exit access control

  - User authentication

#### Extended Application Areas

The system can be adapted for various environments.

  - Smart city public parking areas

  - Airport and railway parking systems

  - Residential societies

  - Industrial and commercial parking

#### Integration of Additional Sensors

Additional sensors can improve system accuracy and functionality.

  - IR sensors for redundancy

  - Camera-based verification

  - Environmental sensors for outdoor monitoring

  - Vehicle count sensors

Overall, these enhancements can significantly improve the effectiveness, scalability, and real-world impact of the Smart Parking System.

### ADDITIONAL IMPLEMENTATION DETAILS

#### Integration of Modules

All system modules are integrated through ESP32 firmware:

  - Ultrasonic sensor data acquisition

  - Threshold-based decision logic

  - Parking status classification

  - Data transmission to server

#### Power Management

  - Low-power operational modes

  - Efficient power utilization

  - Optional battery or solar backup

  - Stable voltage regulation

#### Physical Deployment Design

  - Compact and secure sensor mounting

  - Weather-resistant housing

  - Suitable for indoor and outdoor parking

  - Easy installation and maintenance

#### Calibration of Sensors

- Ultrasonic sensor distance calibration

- Threshold tuning for different vehicle sizes

- Environmental testing for accuracy

- Periodic recalibration support

#### Communication Protocols Used

- GPIO for ultrasonic sensor interfacing

- Wi-Fi (TCP/IP or HTTP) for data transmission

- Server–database communication

FIGURE 5.1 Web-based dashboard visualization

# CHAPTER 6: BIBLIOGRAPHY AND REFERENCES

This chapter lists the technical documents, research papers, and learning resources referred to during the design, implementation, and analysis of the Smart Parking System. These references provided essential guidance on embedded systems, sensor technology, IoT communication, and smart parking architectures.

## BIBLIOGRAPHY

The following resources were consulted to understand system components, technologies, and implementation methods:

- ESP32 Official Documentation and Technical Datasheet

- Ultrasonic Sensor (HC-SR04) Datasheet and Application Notes

- Arduino and ESP32 Programming References

- Research papers on IoT-based smart parking systems

- Online tutorials and technical articles related to IoT and web dashboards

These resources helped in understanding hardware interfacing, real-time data transmission, and system integration.

## REFERENCES

#### ESP32-Based IoT Systems (Core of the Project – MOST IMPORTANT)

S. Li, Y. Zhang, and L. Chen,

“Design and Implementation of an IoT-Based Smart Parking System Using ESP32,”

International Journal of Advanced Computer Science and Applications (IJACSA), vol. 12, no. 8, pp. 456–462, 2021.

This reference was used as the primary guide for ESP32-based IoT architecture and realtime data transmission.

#### Ultrasonic Sensor for Vehicle Detection

R. Kumar and P. Singh,

“Vehicle Detection in Smart Parking System Using Ultrasonic Sensors,”

International Journal of Engineering Research & Technology (IJERT), vol. 9, no. 6, 2020. This paper helped in understanding distance-based vehicle detection and threshold selection.

#### IoT-Based Smart Parking Systems

  - Al-Turjman,

“Smart Parking Solutions Using Internet of Things,”

IEEE Consumer Electronics Magazine, vol. 8, no. 3, pp. 34–39, 2019.

This reference provided insight into large-scale smart parking applications and smart city integration.

#### Web-Based Monitoring and Dashboard Design

M. Patel and S. Shah,

“Real-Time Web Dashboard for IoT Applications,”

International Journal of Computer Applications, vol. 176, no. 15, 2020.

This paper was useful for understanding server–client communication and dashboard visualization.

#### Wireless Communication in IoT Systems

K. Ashton,

“That ‘Internet of Things’ Thing,” RFID Journal, 2009.

This foundational reference explains IoT concepts relevant to real-time monitoring systems.

#### Smart City and Parking Management

N. Lu, C. Lu, and Z. Li,

“Smart City Applications of IoT: A Case Study of Smart Parking,” Sensors, vol. 20, no. 12, 2020.

This paper provided background on smart parking as part of smart city infrastructure.

#### Database and Server-Side Design for IoT

  - Gupta and R. Verma,

“Efficient Data Storage and Processing for IoT-Based Applications,”

International Journal of Information Technology, vol. 11, no. 2, 2019.

This reference helped in understanding backend data handling for real-time systems.

#### Ultrasonic Sensor Application Notes

HC-SR04 Ultrasonic Sensor Datasheet, Technical Application Guide, 2019.

This datasheet was used for sensor calibration, wiring, and distance calculation.

# CHAPTER 7: FUTURE ENHANCEMENTS / RECOMMENDATIONS

## INTRODUCTION

As technology continues to evolve, smart systems are becoming more advanced, efficient, and user-friendly. The proposed Smart Parking System has successfully demonstrated real-time parking monitoring using IoT and embedded systems. However, there is always scope for further improvement and expansion to meet growing user demands and smart city requirements.

Future enhancements focus on improving system intelligence, scalability, automation, and user convenience. By integrating advanced technologies and additional features, the system can be transformed into a more powerful and commercially viable solution.

## FUTURE ENHANCEMENTS

The following enhancements can be implemented to improve the performance and functionality of the system:

1. Mobile Application Integration

- Develop a mobile app for real-time parking updates

- Allow users to check availability remotely

- Provide slot reservation before arrival

- Enable navigation to free parking slots

2. Cloud-Based System

- Store parking data on cloud platforms

- Improve scalability and remote accessibility

- Enable data backup and security

- Reduce dependency on local servers

3. Data Analytics and Prediction

- Analyze parking usage patterns

- Predict peak hours and availability

- Help in better parking management decisions

- Improve system efficiency

4. Automated Payment System

- Enable online payment for parking

- Implement time-based billing

- Reduce manual ticketing process

- Improve user convenience

5. License Plate Recognition

- Use cameras for vehicle identification

- Automate entry and exit process

- Enhance security of parking system

- Maintain vehicle records

6. Improved Hardware Design

- Use weather-proof sensors for outdoor use

- Make system more durable and reliable

- Use compact and efficient hardware setup

- Reduce maintenance requirements

7. Energy Efficiency

- Implement low-power operation modes

- Use solar power for sustainability

- Add battery backup system

- Reduce overall power consumption

8. Integration with Smart City Systems

- Connect with city traffic management systems

- Provide centralized monitoring

- Help reduce urban traffic congestion

- Support smart infrastructure development

9. Additional Sensors Integration

- Use IR sensors for backup detection

- Add camera-based verification

- Improve accuracy and reliability

- Reduce false detection

## CONCLUSION

The Smart Parking System developed in this project provides an effective solution for real-time parking management using IoT technology. While the current system meets all basic requirements, the proposed future enhancements can significantly improve its functionality, scalability, and usability.

By integrating advanced features such as mobile applications, cloud computing, automation, and intelligent analytics, the system can evolve into a complete smart parking solution suitable for large-scale deployment.

These improvements will not only enhance user experience but also contribute to efficient traffic management and smart city development.

## APPENDIX:

CODE:

Arudino IDE code:-

#include <WiFi.h>

#include <HTTPClient.h>

/* -------- PIN DEFINITIONS -------- */

#define TRIG1 5

#define ECHO1 18

#define TRIG2 19

#define ECHO2 21

#define GREEN1 23

#define RED1 22

#define GREEN2 25

#define RED2 26

/* -------- WIFI SETTINGS -------- */

const char* ssid = "Airtel_Patel's home";

const char* password = "Nimisha@123";

String serverName = "hwqttp://192.168.1.6:8080/PS/update.php";

/* -------- VARIABLES -------- */

float distance1;

float distance2;

String status1;

String status2;

String lastStatus1 = "";

String lastStatus2 = "";

unsigned long lastSensorRead = 0;

unsigned long lastSerialPrint = 0;

/* -------- DISTANCE FUNCTION -------- */

float readDistance(int trig, int echo)

{

digitalWrite(trig, LOW);

delayMicroseconds(2);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

long duration = pulseIn(echo, HIGH, 30000);

float distance = duration * 0.034 / 2;

return distance;

}

/* -------- SEND DATA -------- */

void sendData(int slot, String status)

{

Serial.print("Sending to server -> Slot ");

Serial.print(slot);

Serial.print(" : ");

Serial.println(status);

if (WiFi.status() == WL_CONNECTED)

{

HTTPClient http;

String url = serverName + "?slot=" + String(slot) + "&status=" + status;

http.begin(url);

http.setTimeout(200);

int httpCode = http.GET();

Serial.print("HTTP Response: ");

Serial.println(httpCode);

http.end();

}

else

{

Serial.println("WiFi not connected");

}

}

/* -------- SETUP -------- */

void setup()

{

Serial.begin(115200);

pinMode(TRIG1, OUTPUT);

pinMode(ECHO1, INPUT);

pinMode(TRIG2, OUTPUT);

pinMode(ECHO2, INPUT);

pinMode(GREEN1, OUTPUT);

pinMode(RED1, OUTPUT);

pinMode(GREEN2, OUTPUT);

pinMode(RED2, OUTPUT);

Serial.println("Connecting WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED)

{

delay(300);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected");

Serial.print("ESP32 IP: ");

Serial.println(WiFi.localIP());

}

/* -------- LOOP -------- */

void loop()

{

/* ---------- FAST SENSOR READ (80ms) ---------- */

if(millis() - lastSensorRead > 80)

{

distance1 = readDistance(TRIG1,ECHO1);

delayMicroseconds(50);

distance2 = readDistance(TRIG2,ECHO2);

/* SLOT 1 */

if(distance1 < 20)

{

status1 = "occupied";

digitalWrite(RED1,HIGH);

digitalWrite(GREEN1,LOW);

}

else

{

status1 = "empty";

digitalWrite(RED1,LOW);

digitalWrite(GREEN1,HIGH);

}

/* SLOT 2 */

if(distance2 < 20)

{

status2 = "occupied";

digitalWrite(RED2,HIGH);

digitalWrite(GREEN2,LOW);

}

else

{

status2 = "empty";

digitalWrite(RED2,LOW);

digitalWrite(GREEN2,HIGH);

}

lastSensorRead = millis();

}

/* ---------- SEND DATA ONLY IF STATUS CHANGED ---------- */

if(status1 != lastStatus1)

{

sendData(1,status1);

lastStatus1 = status1;

}

if(status2 != lastStatus2)

{

sendData(2,status2);

lastStatus2 = status2;

}

/* ---------- SERIAL PRINT EVERY 1 SECOND ---------- */

if(millis() - lastSerialPrint > 1000)

{

Serial.print("Sensor 1: ");

Serial.print((int)distance1);

Serial.print("cm(");

Serial.print(status1);

Serial.print(") | ");

Serial.print("Sensor 2: ");

Serial.print((int)distance2);

Serial.print("cm(");

Serial.print(status2);

Serial.println(")");

lastSerialPrint = millis();

}

}

xampp ser er cerated codes:-

conn.php:-

<?php

$servername = "localhost";

$username = "root";

$password = "";

$dbname = "parking_system";

$conn = new mysqli($servername,$username,$password,$dbname);

if ($conn->connect_error) {

die("Connection failed: " . $conn->connect_error);

}

?>

index.php:-

<?php

include("conn.php");

$result = $conn->query("SELECT * FROM parking_slots");

?>

<!DOCTYPE html>

<html>

<head>

<title>Smart Parking System</title>

<meta http-equiv="refresh" content="2">

<style>

body{

font-family:Arial;

background:#f4f4f4;

text-align:center;

margin-top:50px;

}

table{

margin:auto;

border-collapse:collapse;

width:70%;

background:white;

box-shadow:0 0 10px rgba(0,0,0,0.2);

}

th,td{

border:1px solid #ddd;

padding:12px;

font-size:18px;

}

th{

background:#2c7be5;

color:white;

}

.empty{

color:green;

font-weight:bold;

}

.occupied{

color:red;

font-weight:bold;

}

</style>

</head>

<body>

<h1>🚗 Smart Parking Dashboard</h1>

<table>

<tr>

<th>Location</th>

<th>Slot Number</th>

<th>Status</th>

<th>Timing</th>

</tr>

<?php

while ($row = $result->fetch_assoc()) {

?>

<tr>

<td><?php echo $row['location']; ?></td>

<td><?php echo $row['slot_number']; ?></td>

<td class="<?php echo $row['status']; ?>">

<?php echo ucfirst($row['status']); ?>

</td>

<td><?php echo $row['timing']; ?></td>

</tr>

<?php

}

?>

</table>

</body>

</html>

<?php $conn->close(); ?>

Update.php:-

<?php

include("conn.php");

if(isset($_GET['slot']) && isset($_GET['status'])){

$slot = $_GET['slot'];

$status = $_GET['status'];

$sql = "UPDATE parking_slots

SET status='$status', timing=NOW()

WHERE slot_number='$slot'";

if ($conn->query($sql) === TRUE){

echo "Updated";

}else{

echo "Error";

}

}else{

echo "Invalid request";

}

$conn->close();

?>